Michael J Curtin

Client Quick Summary

- Release paperwork (NO)

- IF TOTAL LOSS: Fill out CCC/ No call in.

- IF TOTAL LOSS: 3 Salvage Bids required.

- LKQ/Recon/AM over 2 calendar years old and over 24k miles

- Kelly Blue Book (KBB) printout showing the Private Party Value of the unit is required with all files - Repairable or Total Loss. This may include Kelly Blue Book, Edmunds, Carfax, Cars.com

- Always include a refreshed copy of the Advisor Report with all estimates

Client Photo Rules

MCE IS VERY PARTICULAR ABOUT PHOTOS!

- Take as many photos as necessary to show ALL the items listed on your estimate. If you can't see it in the photos, don't write it on your estimate.

- Always take impact height measurements in inches using a Keson Tape Measurer (straight on, level and showing contact with ground)

- Odometer (Mandatory on TL's ) Or oil change sticker showing mileage

- VIN Plate (Both Door and Windshield to confirm vehicle)

- Production date/MFG data plate

- Copy of the vehicle registration if a claimant vehicle

- Unrelated Prior Damage (UPD)

- IF TOTAL LOSS: Numerous photos of the interior showing the condition and options.

Client Estimate/Supplement Release Rules

- NO - Do not release a copy of the estimate to vehicle owner and repair facility.

Client Parts Application Rules

- Utilize LKQ, Aftermarket and Reconditioned parts on all vehicles in excess of 2 years and 24k miles.

- A/M Parts must be CAPA certified or equivalent.

- Glass should always be replaced utilizing the most cost-efficient method.  Including but not limited to Safelite, NAGS, and local aftermarket suppliers.

Client Total Loss Rules

- Fill out the CCC evaluation form

- 3 salvage bids.

- Michael J Curtin uses Kelly Blue Book (KBB) Private Party to value vehicles.  DO NOT USE NADA.  Use the KBB value to determine if the vehicle should be deemed a total loss.

Client Tow Charge Rules

  If towing is related and necessary, include it in the estimate.  Include a copy of the invoice/bill with your upload

Client Supplement Handling Rules

- Repair facility information and the shop Tax ID number must be populated on all supplements. (The TIN must be listed under the license number in CCC)

- Supporting invoices and photos are always to be included on supplement uploads

Client Betterment/Depreciation Rules

- Apply betterment/depreciation on both insured and claimant vehicles on parts with a limited life expectancy; Tires, Batteries, Shocks, Mechanical etc.

- No betterment/depreciation needs to be taken if the replacement part estimated is an aftermarket, reconditioned or LKQ part unless the damaged part was non-OEM. (Not applicable on tires)

- Estimate line note should contain the life expectancy or tire tread depth to document the decision.

Client Documentation Requirements

- Complete the appraisal report in its entirety and provide specific detailed inspection notes.

- Always note in your report the approximate open items.  Provide an approximate supplement amount and an explanation of possible hidden damages and procedures.

- Kelly Blue Book (KBB) printout showing the Private Party Value of the unit is required with all files - Repairable or Total Loss. This may include Kelly Blue Book, Edmunds, Carfax, Cars.com

- Comment in your Appraisal Report the "Approximate Market Value" of the unit.

Client Rates and Sales Tax Rules

- Utilize local prevailing labor rates

- Utilize applicable tax rate

Client Miscellaneous Rules

- Review CCC Advisor Report and correct all errors/exceptions BEFORE locking/uploading estimate.

- If applicable, always supply a UPD estimate.

*MANUAL ENTRY ALLOWANCES/PAINT MATERIAL THRESHOLD:
No paint material cap
(All at sheet metal time unless otherwise noted)
Cover Car (mask for overspray)               $5.00 + 0.3 hour
Color Sand & Buff                                       0.5 hour per EXTERIOR panel @ body rate
Corrosion Protection                                  $10.00
Rust Proofing (Weld through primer)      $10.00
Undercoat                                                     $5.00 + 0.2 hour
Seam Sealer                                                  $8.50 NO LABOR / MATERIALS ONLY
Caulk                                                              $8.50 NO LABOR / MATERIALS ONLY
Gravel Guard (chip resistant)                     0.3 hour 1st panel, 0.2 add’l panels
Complete Detail                                           $100.00 - $150.00 with explanation
Stripe – Tape                                                $10.00 + 0.2 hour per panel
Stripe – Paint                                                $45.00 1st panel, $10.00 add’l panel Maximum: $120.00.
Hazardous Waste Disposal                         $3.50 max per estimate NON TAXABLE
Mount & Balance                                        $ 12.00 maximum
2-Wheel Alignment                                    $ 69.00 maximum – non-shop locale
4-Wheel Alignment                                     $129.00 maximum – non-shop locale
Coolant                                                          $12.00 per gallon
Freon & Oil R-12                                          $25.00 / lb Specify vehicle capacity
Freon & Oil R-134                                        $10.00 / lb Specify vehicle capacity
Recode Lock                                                 $30.00 1st lock, $25.00 add’l lock
Reset Electrical Components                    1.0 hour mechanical
Disable Air Bags                                          0.5 hour mechanical
Frame Set Up/Measure                             2.0 hours at frame rate
Safety Check                                                1.0 hour at mechanical rate.